A.5.M1 • PART 1

# 1. Basic network concepts

Goal: to recognize the types of networks and the basic equipment.

## What is a computer network?

A computer network is a group of two or more devices that are connected so that they can exchange data and share resources, such as files, applications, printers, and Internet access.

| Device A | → | Network | → | Device B |
| --- | --- | --- | --- | --- |

## LAN, WAN and the Internet

| Term | Area | Example |
| --- | --- | --- |
| LAN (Local Area Network) | Small: room, building or nearby buildings | School laboratory |
| WAN (Wide Area Network) | Large: cities or countries | A company's network in many cities |
| Internet | Global interconnection of many networks | The largest WAN |

## The basic network devices

| Device | Role |
| --- | --- |
| NIC / network card | Connects a device wired (RJ45) or wirelessly (Wi‑Fi). |
| Switch | Connects many wired devices on the same LAN. |
| Access Point | Connects wireless devices to the LAN. |
| Router | Connects different networks and forwards the data. |
| Modem | Adapts the signal for connection with the Internet provider. |

Note: At home, modem, router, switch and access point are often found inside the same device.

## Client–Server

| Client requests a service | → | Server provides a service |
| --- | --- | --- |

Example: the browser is a client and requests a web page from a web server.

| Example of a mixed network with wired and wireless devices. | Clients connecting to a server via the Internet. |
| --- | --- |

A.5.M1 • PART 2

# 2. Connections, speed and services

Goal: to choose an appropriate connection medium and distinguish basic Internet services.

## Transmission media

| Medium | How it transfers data | Advantages | Limitations |
| --- | --- | --- | --- |
| Cat 5 / Cat 6 cable | Electrical pulses | Stable connection, low cost | Up to about 100 m, needs RJ45 |
| Optical fiber | Light pulses | Very high speed, many km | Higher cost, more difficult installation |
| Wi‑Fi | Electromagnetic waves | Freedom of movement, easy connection | Interference, obstacles, needs security |

## Data transfer speed

It measures how many bits are transferred every second. The speed depends on the device and the transmission medium.

| 1 Kbps ≈ 1,000 bps | → | 1 Mbps ≈ 1,000,000 bps | → | 1 Gbps ≈ 1,000,000,000 bps |
| --- | --- | --- | --- | --- |

Don't confuse: b = bit, while B = byte. Network speed is usually written in bps, Mbps or Gbps.

## Basic Internet services

| Service | Abbreviation / protocol | What it does |
| --- | --- | --- |
| World Wide Web | WWW / HTTP | Transfers web pages between web servers and browsers. |
| Electronic mail | E‑mail | Transfers electronic messages between users. |
| File transfer | FTP | Sends files from client to server and vice versa. |

## How do I choose a connection?

- Do I need stability and have a short distance? → Network cable.

- Do I need very high speed or long distance? → Optical fiber.

- Do I need mobility? → Wi‑Fi, with a strong password and correct security settings.

Quick check Can you explain why a school laboratory is a LAN? Which medium would you choose to connect two buildings at a long distance?

| Network cables with RJ45. | Optical fibers: transmission with light. | Wireless router / access point. |
| --- | --- | --- |

A.5.M2

# 3. Network settings and diagnostics

Goal: to read the settings of a device and perform basic checks.

## The four basic settings

| Setting | What it means | Example |
| --- | --- | --- |
| IP Address | The unique address of the device on the network. | 192.168.10.23 |
| Subnet Mask | Shows which part of the IP belongs to the network. | 255.255.255.0 (/24) |
| Default Gateway | The IP of the router towards other networks. | 192.168.10.1 |
| DNS Server | Converts a website name into an IP Address. | www.example.com → 93.184.216.34 |

## What is a subnet?

A subnet (subnetwork) is a group of devices with a common network portion. With mask 255.255.255.0 (/24), the first three parts of the IP indicate the network.

| Device | IP Address | Result |
| --- | --- | --- |
| PC A | 192.168.1.10 | Network 192.168.1.0 |
| PC B | 192.168.1.25 | Same subnet — direct communication |
| Laptop | 192.168.2.10 | Different subnet — needs router/gateway |

## Diagnostic commands in Windows

| Command | What it checks | Example |
| --- | --- | --- |
| ipconfig /all | IP, mask, gateway and DNS of the device. | ipconfig /all |
| nslookup | Which DNS is used and which IP corresponds to a name. | nslookup example.com |
| ping | Whether a destination responds on the network. | ping 192.168.1.1 |
| tracert | The intermediate routers (hops) to the destination. | tracert google.com |

## Correct order of checks

| 1. Cable / Wi‑Fi | → | 2. ipconfig | → | 3. ping router | → | 4. ping Internet | → | 5. DNS / tracert |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- ping 8.8.8.8: checks Internet without needing name translation by DNS.

- ping www.google.com: checks both Internet connection and DNS.

- The commands display information; they do not change the settings.

Example of IP Address, Subnet Mask and Default Gateway in two networks.

A.5.M5

# 4. Security and responsible communication

Goal: to recognize protection tools, dangerous messages and safe reactions.

## Three tools — three different roles

| Tool | Checks | Protects against |
| --- | --- | --- |
| Antivirus | Files and applications | Viruses and other malware |
| Firewall | Network traffic and access rules | Suspicious or unauthorized connections |
| Antispyware | Hidden monitoring and data collection | Spyware |

## Social networks and personal data

| What do I share? | → | Who sees it? | → | What remains? |
| --- | --- | --- | --- | --- |

Address, phone number, school, location and photos require caution. Privacy settings limit the audience, but do not always prevent copying. What we publish creates a digital footprint.

## Phishing: four warning signs

| 1 | 2 | 3 | 4 |
| --- | --- | --- | --- |
| Suspicious sender | Pressure or "urgency" | Asks for password or personal details | Strange domain / link |

Safe reaction: Do not click the link. Open the official app or website ourselves and inform a responsible person.

## Grooming and Cyberbullying

| Risk | Signs | What I do |
| --- | --- | --- |
| Grooming | Fake identity, gradual trust, secrecy, request for photos or meeting. | Stop communication and talk immediately to a trusted adult. |
| Cyberbullying | Repeated insults, threats or publishing content without permission. | Do not reply, keep evidence, block, report and ask for help. |

## Final review check

- Explain the difference between antivirus and firewall.

- Mention two signs of phishing.

- State four actions for dealing with cyberbullying.

- Give an example of personal data that should not be published.

The firewall filters traffic between the device and the network.